UNIT 3-MATERIALS

| NATURAL WOODS Woods are classified into two main groups, softwoods and hardwoods. Trees are either coniferous (bears cones and have needle shaped leaves that stay green all year round) or deciduous (has flat leaves that fall in autumn). The timber that comes from the coniferous tree is known as softwood and the timber that comes from deciduous trees is known as hardwood. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Example of SOFTWOOD |  |  |  |  |
| PINE - Is a relatively cheap wood used in the building trade and for furniture. It is pale in colour, quite easy to cut and shape, and machines relatively well. |  |  |  |  |
| Examples of HARDWOODS |  |  |  |  |
| MAHOGANY (caoba) - Is quite expensive and is used for good quality furniture and hardwood windows. It is light brown in colour and more difficult to use compared to pine. |  |  |  |  |
| OAK (roble)- This is an expensive material and is used in for making quality, expensive furniture. |  |  |  |  |
| TEAK (teca) - A hardwood that contains oils which means it is resistant to decay. This is often used to make garden furniture or for wood flooring. |  |  |  |  |
| TASK 1: COMPLETE THE FOLLOWING TABLE ABOUT NATURAL WOODS |  |  |  |  |
| NAME | TYPE | PRICE | MAIN USES | PROPERTIES |
| AMERICAN PINE |  |  |  |  |
| MAHOGANY |  |  |  |  |
| OAK |  |  |  |  |
| EUROPEAN PINE |  |  |  |  |
| TEAK |  |  |  |  |

| TASK 2: look for pictures from magazines about wood objects. What type of wood are they made of? |  |
| --- | --- |

| MANMADE BOARDS These are boards that are made in factories from materials such as wood chippings or dust. The sheets are available (disponible) in standard sizes 2440 mm by 1220 mm and range from 1.5 mm to 25 mm thick. |  |
| --- | --- |
|  | BLOCKBOARD – It is made from softwood strips bonded with an adhesive and covered with a veneer (lámina) of plywood on either side. Used as a building material and for furniture manufacturing (kitchens shelfs cupboards / bedrooms). |
|  | CHIPBOARD (AGLOMERADO)- It is made from small chips of wood bonded together with a resin and formed into sheets by compression. It is not as strong as plywood and block board but it is not expensive. It is used in internal parts of furniture. Chipboard is often covered with plastic or wood veneers and used in cheap furniture manufacturing. |
|  | MEDIUM DENSITY FIBRE BOARD (MDF) (DM)- This board is made from wood dust and resin, and then pressed into a board. This material can be worked, shaped and machined easily. Easy to paint without primer. It is used in some parts of buildings (closets, wardrobes, cabinets, cupboards..) and furniture manufacturing. It is cheap. |
|  | PLYWOOD (CONTRACHAPADO) – This is made from sheets of timber with each grain layer is placed at right angles (90º) to each other and bonded together by resin and pressure. It is stronger than the rest of manmade boards. It is used in kitchen furniture, covered by decorated plastics veneers. |

| TASK 3: COMPLETE THE FOLLOWING TABLE ABOUT MANMADE BOARDS. |  |  |  |
| --- | --- | --- | --- |
| NAME | IT IS MADE FROM ... | MAIN USES | OTHER CARACTERISTICS |
| BLOCKBOARD |  |  |  |
| CHIPBOARD |  |  |  |
| MDF |  |  |  |
| PLYWOOD |  |  |  |

| WOOD PROPERTIES |  |
| --- | --- |
| DENSITY | It is the relatioship between mass and volume of the sample. Al woods are less dense than water, so they can float in water. |
| HARDNESS | It is the material´s ability to penetrate softer materials. An object made of a hard material will scratch an object made of a softer material. |
| TENSILE STRESS (R. a la tracción) | It is the mechanical property of resisting pulling forces (pull= tirar). Wood is relatively weak in tension perpendicular to the grain but it is very strong in tension parallel to wood fibers. |
| COMPRESSION STRESS | It is the mechanical property of resisting pushing forces. (push=empujar). The values are different when the forces are perpendicular or parallel to the wood fibers. |
| FLEXIBILITY | It is the property of an object of being bent under certain circumstances. |
| DURABILITY | It is the ability of resisting the action of living organisms, moisture, etc. |
| THERMAL CONDUCTIVITY | It is the ability of a material to transfer heat. Woods do not conduct heat. So they are THERMAL INSULATORS. |
| ELECTRICAL CONDUCTIVITY | It is the ability of a material to allow the flow of electricity. Woods do not conduct electricity (when it is dry), so they are ELECTRICITY INSULATORS. |

TIMBER

Task 4: Answer the following questions

1. How many types of natural timber are there? Name them.

2. Explain three disadvantages of natural timbers.

3. Why do we say that hardwoods are slow growing?

4. Why are hardwoods more expensive than softwoods.

Task 5: Match the elements on the right with the right element on the left.

| OAK |  | SOFT AND LIGHT |
| --- | --- | --- |
| BALSA |  | OPEN GRAINED |
| ASH |  | STRONG AND HARD |
| TEAK |  | FLEXIBLE |
| BEECH |  | VERY HEAVY |
| MAHOGANY |  | FAIRLY STRONG |

TASK 6: Answer the following questions.

1. Why do we say that softwoods are fast growing?

2. Why do we say that softwoods are open grained?

3.Why do you think that softwoods are cheaper then hardwoods?

TASK 7: Guess the name of the wood

1. I am fairly strong, straight-grained but knotty. You can find me easily. My name is ………………………..

2. I am almost knot free. I am strong and durable and very expensive to be a softwood. My name is ………………..

3. I am quite dense and brown or red coloured. My name is ……………………………

4. I am knot free, light and silky. I can be placed outside because I can resist weather and insects. My name is ……………………………..

TASK 8: MANUFACTURED BOARDS

1. Write the main advantages of manufactured boards.

2. Write the characteristics of Plywood and the thickness available.

3. Write the characteristics of chip-board and the thickness available.

4. Write the characteristics of block board and the thickness available.